# ECE4301.01                                           SHA-1 & SHA-256 on Raspberry Pi 5

Team Members:

Caleb Jala-Guinto – Implemented Rust CLI for SHA-1 and SHA-256 hashing
Isabel Warth – Conducted benchmarking, collected data/throughput plots, formatted figures
Manuel Alvarado – Compiled report and integrated results

## Introduction

The goal of this project is to evaluate the performance of SHA-1 and SHA-256 hashing algorithms on a Raspberry Pi 5 platform. The benchmarking study compares two Rust implementations, a software-only variant and a hardware-accelerated variant leveraging Armv8 SHA instructions, against the native system engine provided by Linux. Through careful measurement of hashing throughput across a range of buffer sizes, this study gave us insights into the advantages of hardware acceleration and informed recommendations for cryptographic processing in embedded or constrained environments.

## System Configuration and Software Environment

All benchmarking was performed on a Raspberry Pi 5 running Debian GNU/Linux 12 (Bookworm) with Linux kernel 6.12.47+rpt-rpi-2712 on an aarch64 architecture. This platform supports the Armv8 SHA extensions required for the hardware-accelerated tests.

The cryptographic implementations were built using the Rust toolchain installed directly on the Pi. The project depended on the following crates:

- sha1 v0.10.6
- sha2 v0.10.9

Both were used within the project crypto-on-chip v0.1.0.

To ensure consistent benchmarking conditions, the CPU frequency governor was set to performance mode, and all runs were pinned to a single core using taskset. These measures minimized variability and ensured reproducible comparisons between the software-only Rust implementation, the hardware-accelerated version, and the system engine.

Benchmark Methodology

### Rust Implementations

Two Rust binaries were benchmarked:

- Software-Only (sha_rust_cli_soft) — This implementation relies entirely on Rust software routines without any platform-specific assembly optimizations.

- Hardware-Accelerated (sha_rust_cli_accel) — This implementation leverages the Armv8 SHA extensions available on the Raspberry Pi 5 for increased throughput.

Benchmarking consisted of hashing buffers of varying sizes (1 KiB, 8 KiB, 64 KiB, and 1 MiB) across multiple trials (≥5 per buffer size) to obtain statistically meaningful results. Wall-clock time was recorded for each trial, and throughput was subsequently calculated in megabytes per second.

### System Engine Baseline

For comparison, the platform-native engine was measured using either kcapi-speed or OpenSSL’s speed utility. Both SHA-1 and SHA-256 were benchmarked across the same message sizes to facilitate direct comparison with the Rust implementations. Raw console output was preserved for later analysis and inclusion in supplementary materials.

### File-Based Verification

To validate correctness and demonstrate real-world behavior beyond synthetic buffer benchmarks, both Rust binaries (soft and accel) were evaluated using five test images captured with the Raspberry Pi camera. Each image shows different numbered hand signals, just to make them visually distinct inputs while still being small, practical test files.

For each image, both binaries computed SHA-1 and SHA-256 digests and recorded wall-clock hashing time. The digests produced by the software-only and hardware-accelerated versions were identical for all images, confirming functional correctness across both implementations. The hardware-accelerated binary consistently showed much lower per-file latency, reducing hashing time from roughly 5.1–5.6 ms (soft) to 1.28–1.44 ms (accel).

A summary of the recorded results is shown below

| ### Accelerated Implementation (Armv8 SHA) (from demo_accel_files.txt) - img1.jpg — wall: 0.001370 s - img2.jpg — wall: 0.001397 s - img3.jpg — wall: 0.001384 s - img4.jpg — wall: 0.001284 s - img5.jpg — wall: 0.001443 s | ### Software-Only Implementation (from demo_soft_files.txt) - img1.jpg — wall: 0.005074 s - img2.jpg — wall: 0.005310 s - img3.jpg — wall: 0.005539 s - img4.jpg — wall: 0.005120 s - img5.jpg — wall: 0.005662 s |
| --- | --- |

Across all five verification files, the hardware-accelerated implementation demonstrated a consistent 3.7×–4.3× reduction in hashing time, matching the performance trends observed in the randomized-buffer benchmarks. This validation step confirms not only correctness but also the practical performance benefits of Armv8 SHA acceleration when hashing real user data, such as images captured directly on the Raspberry Pi 5.

## Results and Data Presentation

### Throughput Analysis  Throughput versus buffer size for Rust-soft, Rust-accel, and the system engine

Observations:

Across both SHA-1 and SHA-256, all implementations show the same general trend with respect to buffer size, throughput improves significantly from small buffers (2¹¹ bytes) to medium buffers (2¹³ bytes). After that point (2¹³ → 2¹⁹), performance plateaus, showing only small incremental gains.

Low variance across repeated trials for both SHA-1 and SHA-256. No major noise or instability in the measurements. Performance is deterministic at these buffer sizes because: CPU frequency remains stable, L1 and L2 cache effects are consistent, Data fits comfortably in memory (no paging or contention).

### Discussion and Analysis

- Hardware Acceleration Benefits
 The Armv8 SHA extensions provide a significant performance boost by replacing instruction-heavy software hashing with dedicated hardware instructions. In our results, the accelerated Rust binary consistently delivered multi-fold throughput gains, especially on larger buffers where overhead no longer dominates. This matches expectations: once running at steady state, hardware SHA maintains a high, predictable block-processing rate, while software implementations hit instruction-count and ALU bottlenecks.

- Small vs. Large Buffer Behavior
 For small inputs, performance is limited by fixed overheads, setup, memory access, cache warm-up, so both Rust binaries behave similarly. Throughput here reflects latency, not raw speed. As buffer sizes increase, overhead is amortized, allowing the accelerated version to scale sharply and show its largest speedups. This matters for real workloads: small-packet traffic sees limited benefit, while bulk data workloads (file hashing, logging, image streams) see substantial acceleration.

- Comparison with the System Engine
 The Linux hashing engine typically remains the fastest overall due to hand-tuned assembly and minimal overhead, though the hardware-accelerated Rust version comes surprisingly close while staying fully in user space. The software-only Rust build trails both, limited by higher instruction counts per block and greater cache pressure, but remains portable and consistent.

- Practical Application Recommendation
 For large, continuous data streams, the hardware-accelerated Rust build is the clear best choice for strong throughput and better CPU efficiency. For small, latency-sensitive messages or environments requiring portability, the software-only version is sufficient. When maximum performance is required and external dependencies are acceptable, the system engine offers the highest, most stable throughput.

### Raw Benchmark Files

Rust Benchmark CSVs
 The following CSV files contain the raw throughput measurements for SHA-1 and SHA-256 across all buffer sizes and trials:

rust_soft.csv – Software-only Rust implementation
rust_accel.csv – Armv8 SHA-accelerated Rust implementation
speedups.csv – Calculated acceleration factors (accel_over_soft) per algorithm and buffer size

Example entries (throughput in MiB/s):

| Algorithm | Build | Buffer Size | Trial 1 | Trial 2 | Trial 3 | Trial 4 | Trial 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SHA-1 | Soft | 1 KiB | 183.7 | 252.3 | 252.3 | 252.3 | 252.3 |
| SHA-1 | Accel | 1 KiB | 421.8 | 824.1 | 811.1 | 811.1 | 811.1 |
| SHA-256 | Soft | 1 MiB | 146.3 | 162.9 | 163.5 | 163.6 | 163.0 |
| SHA-256 | Accel | 1 MiB | 894.2 | 886.3 | 894.3 | 894.3 | 891.0 |

Acceleration Factors (accel_over_soft):

| Algorithm | Buffer Size | Speedup (×) |
| --- | --- | --- |
| SHA-1 | 1 KiB | 3.08 |
| SHA-1 | 8 KiB | 3.36 |
| SHA-1 | 64 KiB | 3.34 |
| SHA-1 | 1 MiB | 3.37 |
| SHA-256 | 1 KiB | 5.89 |
| SHA-256 | 8 KiB | 6.15 |
| SHA-256 | 64 KiB | 6.20 |
| SHA-256 | 1 MiB | 5.58 |

System Engine Logs (OpenSSL)
Raw outputs from openssl speed -evp for SHA-1 and SHA-256 on the Raspberry Pi 5:

engine_openssl_sha1.txt		engine_openssl_sha256.txt

| Buffer Size | Throughput (MiB/s) |
| --- | --- |
| 1 KiB | 1039.2 |
| 8 KiB | 1405.0 |
| 64 KiB | 1465.5 |
| 1 MiB | 1474.9 |

| Buffer Size | Throughput (MiB/s) |
| --- | --- |
| 1 KiB | 1077.0 |
| 8 KiB | 1455.3 |
| 64 KiB | 1524.1 |
| 1 MiB | 1534.1 |

- All Rust benchmarks were pinned to a single core and measured over ≥5 trials per buffer size.
- Throughput is reported in MiB/s.
- The included speedup table shows how much faster the Armv8-accelerated Rust builds are compared to software-only Rust.
- Our raw CSVs and OpenSSL logs are provided in the project submission.